Communication-Aware Policy Optimization with

Adaptive Topology Control for Multi-UAV

Cooperative Navigation

Abstract—Multi-UAV   systems   require   efficient   cooperation under  limited  communication  resources,  including  bandwidth, energy, and latency constraints. Existing multi-agent  reinforce- ment  learning   methods  typically  assume  dense  or  geometry- predefined messaging and therefore do not treat communication itself as  an  optimization  target.  We  propose  AC-DSGF,  which learns  a  task-dependent  communication  topology  jointly  with residual  guidance  under  an  explicit  communication  cost  and Top-K budget. On 16-UAV navigation with five seeds, AC-DSGF achieves comparable task performance to DSGF while reducing communication  overhead  by  over  one  order  of magnitude  and improving the Communication Efficiency Index. Additional bud- get, silence-collapse, trigger, and packet-loss studies indicate task- driven sparsity and graceful degradation under unreliable links, supporting bandwidth-constrained cooperative UAV deployment.

Index  Terms—Multi-UAV  systems,  multi-agent  reinforcement learning, communication-efficient cooperation, dynamic graphs, residual guidance.

# I.  INTRODUCTION

a)  Engineering reality.:  In multi-UAV cooperation, every message  costs  bandwidth,  energy,  and  latency.  Controllers that  optimize  only  local  policies  under  an  implicit  “unlim- ited radio”  assumption  therefore  leave  a  first-class  resource unoptimized: communication itself.

b)  Limitation of prior MARL guidance.:  MAPPO-style CTDE stabilizes decentralized control  [1], and graph guides (e.g., GAT) encode spatial relations for early coordination. A common pipeline is

policy  →  (predefined) communication  →  environment.

Topology is treated as given—by a radius, a dense graph, or a hand schedule—so agents cannot decide whether an exchange is currently needed.

c)  Our shift.:  We invert that dependency and learn topol- ogy as part of decision making:

environment  →  learn communication topology   →  policy.

Unlike  previous  approaches  that  assume  commu- nication  topology  is predefined,  we  treat topology selection as a learnable decision process.

Concretely, AC-DSGF answers: when to talk, with whom, and how much under a budget (Q1–Q3).

d)  Method sketch.:  AC-DSGF extends DSGF v2 with a

soft gate gtij , a Top-K budget layer AACt = A Θ EK , and the

joint objective E[ε rt] - λcE[ε Ct], while residual guidance a = π(o)+β∆(Φ) keeps communication assistive rather than dominant (Fig. 1).

Fig. 1.  Motivation: without residual decoupling, guided MARL can collapse under long-horizon optimization (4-UAV ablation; Table VI). AC-DSGF keeps residual guidance while adding budget-aware communication.

e)  Claim.:  We  do  not position AC-DSGF as a Success maximizer. Experiments  show it achieves  comparable  task performance to DSGF while reducing communication over- head by over one order of magnitude (Table V).

f)  Contributions.:

1)  Learnable communication topology: a soft gate gtij  = σ(fθ (hi , hj , dij , qij )) on radius support.

2)  Budget-aware    edge    selection:    Top-K    adjacency AACt = A Θ EKt under J = E[ε r] - λcE[ε C].

3)  Residual  policy  decoupling:  a  =  π(o) + β∆(Φ)  so guidance assists rather than dominates execution.

4)  Evaluation at N=16 (five seeds) covering comparison, budget, silence-collapse, trigger/stability, compute cost, and packet-loss robustness.

# II.  RELATED WORK

We  organize  prior  art  into  three  lines  and  highlight  the residual gap that AC-DSGF targets: joint optimization of task return, adaptive topology, and explicit communication budgets.

## A.  Multi-Agent RL for UAV Swarms

Centralized Training with Decentralized Execution (CTDE) methods  such  as  MAPPO  stabilize  cooperative  control  by sharing  a  critic  during  training  while  keeping  actors  local at  execution time  [1],  [2],  [3].  Extensions  to  aerial  swarms demonstrate  strong  local  navigation  and  collision  handling under  partial  observability   [4],   [5].  However,  most  CTDE pipelines optimize policy return under an implicit assumption of  sufficient  communication  resources:  either  agents  are treated  as  independent  given  a  shared  training  signal,  or coordination relies on unlimited centralized features that do

not survive bandwidth, latency, and energy limits on real radio links.

Gap  1.  Existing  MARL  methods  for  UAV  coop- eration  mainly  optimize  policy  performance  while assuming sufficient communication resources.

## B.  Graph-based Multi-Agent Interaction Modeling

Graph neural networks and attention mechanisms inject re- lational inductive bias into MARL [6], [7], [8], [9]. GAT-style guides  and  graph Transformers  adaptively weight neighbors and can capture time-varying spatial relations for swarm nav- igation. Yet attention support is typically defined by geometry (e.g., communication radius) or is fully dense. Dense message passing incurs O(N2 ) cost that grows quickly for N≥16–32 UAVs, while binary radius graphs still exchange messages on all feasible edges regardless of task urgency. Our prior DSGF improves sparsity via quality weighting and residual guidance, but still activates the full geometric support whenever Aij =1.

Gap 2. Graph MARL models interaction topology but  lacks  explicit  communication  budget  opti- mization—topology adaptation is not equated with bandwidth-aware economy.

## C.  Communication-Efficient MARL

A parallel literature  studies message pruning, gated com- munication,  dropout,  and bandwidth-aware  objectives  to re- duce  exchange  volume   [10],   [11],   [12],   [13],   [14],   [15], [16]. Many approaches, however, rely on random or heuristic suppression of messages, hand-designed schedules, or discrete “talk/silence” decisions that are weakly coupled to continuous residual guidance for control. Under severe budgets, random pruning  can  discard  coordination-critical  links;  conversely, over-aggressive silence collapses information flow.

Gap 3. Prior communication-efficient MARL often drops messages rather than learning when commu- nication is necessary for the ongoing task under a quantified budget.

a)  Positioning.:   Different  from  the   above,  AC-DSGF jointly  considers  (i)  task performance,  (ii)  learned  commu-

nication  topology  adaptation  via  gates  gtij ,  and  (iii)  strict

communication-budget  constraints  through  Ccomm   =  Σ gij and optional Top-K scheduling, while retaining residual DSGF guidance  for  long-horizon  stability.  It  is  therefore  not  a mere  DSGF  model  with  an  added  L2-style  penalty,  but  a communication-constrained coordination stack whose sparsity is task-dependent (validated in Secs. VI-D–VI-E).

# III.  PROBLEM FORMULATION

## A.  Multi-UAV Cooperative Navigation

Consider a team of N UAVs performing cooperative naviga- tion in a continuous 2D workspace under partial observability. At  discrete  time  t,  agent  i  ∈  {1,..., N}  receives  a  local observation

oti = [pti, vti, gti, ℓti] ∈ Rdo ,                     (1)

where  pti, vti   ∈  R2   are  position  and  velocity,  gti  is  the

goal-relative  vector,  and  ℓti   denotes  onboard  range  sensing.

Each  agent  executes  a  continuous  action  ati   ∈  A  under  a

decentralized policy with centralized training (CTDE).

The team  seeks to maximize expected cumulative reward under communication cost,

N

where  rti   =  rgoali(t) + rcolli(t)  and  Ct   is  defined  below.

When λc =0, Eq. (2) reduces to standard return maximization (baselines).  For  AC-DSGF,  λc >0  makes  communication  a first-class training signal.

## B.  Communication Constraint

Airborne links are range-limited and capacity-constrained.

Two agents may exchange messages only if ∥pti − ptj ∥ < Rc ,

inducing a feasible mask At  ∈ {0, 1}N ×N . We further impose

a soft communication budget through a learnable gate gtij   ∈

[0, 1]. Unless stated otherwise, the communication cost used in tables is the time-averaged soft gate mass

(3)

For un-gated baselines (GAT/DSGF), we report the analogous

average  number  of  active  geometric  edges   t Σij Atij

(or  quality-weighted  support)  so  that   Comm  columns  are comparable as message intensity, not raw radio packet counts. The Table I AC-DSGF value C≈0.43 is therefore the mean soft mass under Eq. (3) over the 200-ep evaluation—not an undefined unit. Eq. (2) pairs this C with task return via λc.

Hard budgets additionally restrict support via AACt = At ⊙EKt

(Sec. IV-B).

## C.  Metric: Communication Efficiency Index

To compare methods under different messaging intensities, we report

CEI  where  S  is   the  evaluation  success  rate.  MAPPO  has  no explicit graph messaging (Ccomm =0) and CEI is marked as not applicable. Our primary claim is not that AC-DSGF max- imizes S unconditionally, but that it preserves comparable S while substantially reducing Ccomm  relative to fixed-topology guidance.

NOTATION

# IV.  PROPOSED METHOD: AC-DSGF

We   propose   Adaptive    Communication-aware   Dynamic Sparse Graph Fusion (AC-DSGF). Unlike post-hoc message pruning,  AC-DSGF jointly  learns  the  control  policy  and  a task-dependent  communication  topology  under  an  explicit cost.  Fig. 2  shows  three  layers:  (i)  observation  embedding, (ii)  adaptive  communication  (gate  +  budget),  (iii)  residual DSGF policy.

TABLE I

SUMMARY OF KEY SYMBOLS . SOFT GATES gtij∈[0, 1] ARE CONTINUOUS

(PROBABILITY-LIKE INTENSITIES), NOT HARD BINARY EDGES; BINARY

DECISIONS APPEAR ONLY AFTER TOP-K WHEN A HARD BUDGET IS

ENFORCED .

| Symbol | Meaning |
| --- | --- |
| N oti hti At / Atij qtij gtij K / Ki AAtC etij Gt = (V, Et) αtij Ct / C λc Φt i β(t) ati | Number of UAVs (agents) Local observation of agent i at time t Latent embedding of oti Feasible radius topology (binary mask) Communication quality score Soft communication gate in [0, 1] (not binary) (Per-agent) communication budget / Top-K width Budgeted adjacency At Θ EKt Effective edge weight for attention Communication graph at time t Attention weight on edge (i, j) Instantaneous / time-averaged soft communication cost Communication cost weight in the joint objective Residual guidance feature Residual mixing schedule Action: π(oi) + β∆(Φi) |

Fig. 2.   Framework  of AC-DSGF: adaptive communication (gate + budget) defines the topology consumed by residual DSGF guidance, rather than post- hoc pruning.

## A.  Joint Communication–Control Objective

Let rt  = Σi r denote the team reward at time t and

the soft communication mass. AC-DSGF optimizes

i.e., communication is optimized together with policy learning. This  differs  from  training  a  dense  communicator  and  then pruning messages at deployment time.

## B.  Adaptive  Communication Graph

a)  Feasible mask and quality.:  The radius mask  Atij  = ⊮~[∥pti — ptj ∥  <  Rc] and  quality  qtij    ∈  (0, 1)  yield  support Atij = Atijqtij .

b)  Gate  (learned  topology).:   Let  hti   =  ψ(oti)  and  let

ρtij  encode an interaction-risk feature (relative closing speed

/  proximity  prior;  implemented  via  the  edge-state  channels

alongside dtij and qtij ). The communication gate is

gtij  = σ(Wg [hti; htj ; dtij ; ρtij]) · Atij ∈ [0, 1],        (7)

where Wg  denotes the parameters of a lightweight MLP (bias

omitted). Unlike random dropout, gtij  is a learned function of

agent states and risk—empirically aligned with proximity risk (Sec. VI-E).

c)  Hard budget  ⇒ AC adjacency.:  Given budget ratio ρ ∈ (0, 1],

Ki  = ⌈ρ|Ni |⌉,       EKt = TopKKi ({gtij }j∈Ni ),     (8)

and the adaptive communication adjacency

AACt = At ⊙ EKt,        etij = AAijC qtij gtij .           (9)

Training may use soft g with the cost in Eq. (6); evaluation can enforce hard Top-K without retraining (budget sweep).

d)  Difference from pruning.:  Message pruning typically removes links after (or independently of) policy learning. AC- DSGF  changes the  information  topology inside the  forward pass: setting g=A (“no-budget”) restores near-full local broad- cast and inflates Ct  by  over three orders of magnitude with only a small Success change (Sec. VI-D). Hence AC-DSGF is not “DSGF + ℓ2  regularization,” but a gated topology learner under a joint objective.

## C.  Residual Spatial–Temporal Guidance

Sparse attention aggregates neighbors under et:

αtij  ∝ exp(f(hti, htj)) etij ,        h  (10)

followed by zti = GRU(h′it , zti−1) and guidance Φti . Actions

use residual decoupling

ati ∼ πθ( · | oti)  + β(t)∆(Φti),                 (11)

so coordination assists rather than dominates execution (Ta- ble VI: 9.27% → 0.22% without residual).

## D.   Difference from Conventional Communication Learning

Table II  contrasts  AC-DSGF  with  common  alternatives. Key distinction: the communication topology is optimized as a  decision  variable  rather  than  treated  as  a  fixed  network structure.  Joint  optimization  (Eq.  (6))  and  the  explicit  map A  →  g  →  AAC    →  e  make  gating  part  of  the  forward computation,  not  an  ℓ2 -style  afterthought  on  an  otherwise dense GAT/DSGF stack.

TABLE II

Algorithm 1 AC-DSGF Training

HOW COMMUNICATION IS TREATED DURING LEARNING .                     Require:  Observations {oti}, radius Rc , budget ratio ρ (or K),	Method                                     Communication mechanism                                  weight λc

Require:  Policy parameters θ, gate parameters ϕ (collectively

DPospt-ohtc // rapdnomalty -pruonlningy      StoCoasticraint epleptioliedn oafmre(ssagor areond) a fixed policy graθphin text)

GAT / radius graphs               Attention on a fixed geometric adjacency             Ensure:  Updated θ,ϕ

AC-DSGF                                Task-driven gate + budget; topology is co-learned1:   for each training iteration do

Fig. 3.   Gate stability: mean±std communication mass Ct  and mean  |∆Et| over episodes. Sparse AC graphs are dynamic yet non-chattering.

## E.  Gate Stability (non-random switching)

A  potential  concern  is  that  sparse  gates  might  chatter and destabilize training. Using frozen AC-DSGF rollouts, we measure soft cost variance Var(Ct ) and the edge-change rate

∆Et  = I |Et | — |Et- 1 | I .                      (12)

Empirically  (Fig. 3;  12  episodes),  Var(Ct )≈3 × 10-3   and >99% of steps  have  ∆Et =0:  Ct   varies  smoothly  with  the task phase while edges rarely chatter. Communication adapts when proximity risk changes (Sec. VI-E), rather than randomly toggling.

## F.  Optimization

Actors   use   clipped   PPO   +   GAE   with   the   AC   cost term   for    λc =10-3     (frozen).   Algorithm   1    makes   the Gate→Budget→Policy order explicit. Computational analysis is given in Sec. IV-G.

## G.   Computational and Communication  Complexity

Let  N be  the  number  of UAVs,  d the hidden  width, Ni the feasible neighbors of agent i under radius Rc , and  k =	i |Ni |  the  average  degree  (k  ≪  N  in  sparse  swarms). Let  Ki   =  ⌈ρ|Ni |⌉  denote  the  Top-K  budgeted  degree  and K i Ki  the average budgeted degree (K ≤ k).

a)  Dense  /  GAT  baselines.:   Full  pairwise  messaging or  dense   attention  costs   O(N2 d)  per  layer.  Even  with   a binary radius mask, GAT-style aggregation that treats all in- radius neighbors equally still processes Θ(Nk) edges; under moderate Rc  this rapidly approaches quadratic growth in N.

b) AC-DSGF.:  Gate evaluation runs on the feasible edge set:  O(|E|d) = O(Nkd). Top-K budgeting per agent costs O(k log k)  (or  O(K log k)  with  partial  selection),  totaling O(Nk log k). Sparse attention then operates on the budgeted support of size O(NK):

O(NKd + Nd)   (attention + GRU),           (13)

2:        Collect rollouts with decentralized actors

3:        for each timestep t in batch do

4:            Embed  hti  ←  ψ(oti);  build  candidate mask  At  and

quality qt

5:             Gate: gtij  ← σ(Wg [hi ;hj ;dij ;ρij ]) · Atij

6:             Budget: EKt ← TopKKi ({gtij });  AACt ← At ⊙ EKt

7:           Build etij ← AAijC qtij gtij ;  Ct  ←Σij gtij

8:             Aggregate  neighbors  with   sparse  attention  on   et ;

GRU → Φti

9:             Policy: sample ati ∼ πθ ( · | oti) + β(t)∆(Φti)

10:        end for

11:       Update θ,ϕ by PPO/GAE loss plus λcE[Σt Ct]

12:  end for

| TABLE III COMPLEXITY AND TYPICAL COMMUNICATION INTENSITY (CONCEPTUAL) . K IS THE AVERAGE BUDGETED DEGREE; k IS THE AVERAGE RADIUS DEGREE . |  |  |  |
| --- | --- | --- | --- |
| Method | Communication | Aggregation | Complexity |
| MAPPO | none | — | O(Nd) |
| GAT / Transformer | high | dense / full | O(N2 d) |
| DSGF | medium (all radius links) | sparse k | O(Nkd + Nd) |
| AC-DSGF | low (Top-K gates) | sparse K≤k | O(NKd + Nd) |

which approaches linear scaling in N when K is held approx- imately constant by the budget ratio. Because K ≪ N under tight  budgets,  the  relative  advantage  versus  O(N2 d)  dense attention widens as team size grows. Communication volume itself is bounded by Σi Ki  = O(NK), whereas GAT/DSGF without gating use O(Nk) open geometric edges.

Pairwise  distance  masking  remains   O(N2 )  and  is  shared by  all  geometric  graph  methods;  the  practical  gain  of  AC- DSGF is in attended edges and transmitted soft mass under K ≪ N. Fig. 6 illustrates communication versus team  size. Table IV reports CPU wall-clock per decision step at N=16 (gate overhead is small relative to learning benefits of sparse messaging).

# V.  EXPERIMENTAL SETUP

## A.  Environment and Protocol

All  formal results use the VMAS  continuous multi-agent navigation scenario. Unless stated otherwise:

•   Training:  102,400 frames; evaluation: 200 episodes.

•   Main table seeds:  {42, 3407, 2026, 1234, 8888} (N=16).

•   Horizon   H=128;  success  threshold  0.3  world   units; Rc =0.5.

•   DSGF     v2     backbone     and     AC     hyperparameters are               frozen             (dsgf_v2_frozen.yaml, ac_dsgf_16uav.yaml).

TABLE IV

PER-STEP INFERENCE COST ON CPU (N=16, 50 TIMED STEPS AFTER

WARMUP) . AC-DSGF ADDS A LIGHTWEIGHT GATE; SOFT EDGE MASS

REMAINS FAR BELOW DENSE GUIDES .

| Method | Inference (ms/step) | Soft edges (last) |
| --- | --- | --- |
| GAT | 1.66 | — |
| Transformer | 1.69 | — |
| DSGF | 3.35 | — |
| AC-DSGF | 3.94 | ≈ 0.006 |

TABLE V

TASK PERFORMANCE UNDER COMMUNICATION CONSTRAINTS (N=16, 5

SEEDS , 200-EP EVAL) . AC-DSGF MAINTAINS COMPARABLE TASK

COMPLETION WITH SUBSTANTIALLY REDUCED COMMUNICATION

OVERHEAD (HIGHER CEI) . MEAN±STD .

Collision (%)

Method

Comm

CEI

Success (%)

| MAPPO GAT Transformer DSGF AC-DSGF | 2.40 ± 0.90 2.04 ± 0.90 0.70 ± 0.16 4.01 ± 1.68 3.95 ± 0.83 | 32.3 ± 1.9 29.6 ± 1.0 30.8 ± 3.0 31.5 ± 3.3 30.8 ± 1.4 | 0.00 ± 0.00 38.80 ± 1.99 240.00 ± 0.00 39.27 ± 1.92 0.43 ± 0.09 | — 0.0005 ≈ 0 0.0010 0.0913 |
| --- | --- | --- | --- | --- |

TABLE VI

DSGF RESIDUAL ABLATION AT N=4 (MECHANISM TEST; not TABLE I

SCALE) .

| Variant | Success (%) |
| --- | --- |
| w/o Dynamic Graph | 0.28 |
| w/o Temporal | 0.86 |
| w/o Residual | 0.22 |
| Full DSGF | 9.27 |

TABLE VII

SUCCESS (%) UNDER COMMUNICATION BUDGET RATIO ρ (N=16,

SEED 42, 64-EP DIAGNOSTIC EVAL; QUALITATIVE CONSISTENCY WITH

TABLE I CEI RANKING) . SOFT-COST SCALES DIFFER FROM TABLE I; WE

REPORT TRENDS , NOT ABSOLUTE % INTO THE MAIN CLAIM .

| Budget ρ | GAT | DSGF | AC-DSGF |
| --- | --- | --- | --- |
| 100% | 21.3 | 21.7 | 27.1 |
| 50% | 18.2 | 21.6 | 25.2 |
| 10% | 16.9 | 21.7 | 24.2 |

## B.  Baselines

MAPPO  (no graph messaging);  GAT-MAPPO  (binary ra- dius graph + alignment reward); Transformer / full attention; DSGF  v2  (quality-weighted  sparse  graph  +  residual);  AC- DSGF (DSGF v2 + learned gate + λcCcomm ).

## C.  Metrics

Success S, collision rate, episodic reward, communication cost Ccomm  (Eq.  (3)), and CEI = S/(Ccomm + ε) (Eq. (4)). Demo videos are qualitative and not used as Table metrics.

## D.  Experiment Map

1)  Table V: task performance under communication con- straints (N=16).

2)  Table VI: residual ablation (N=4).

3)  Table VII / Fig. 4: communication budget analysis.

4)  Table VIII: silence-collapse validation.

5)  Fig. 5 / Fig. 3: behavior & gate stability.

6)  Fig. 6 / Fig. 8: scalability & Pareto.

# VI.  RESULTS

## A.   Task Performance under  Communication Constraints

AC-DSGF achieves Success comparable to DSGF (3.95% vs. 4.01%)  while  reducing  communication  from  39.27  to  0.43 (~91×) and lifting CEI by nearly two orders of magnitude (Fig. 8).  We  deliberately  do  not  claim  Success  leadership: the value proposition is communication-efficient cooperation under a realistic swarm constraint.

## B. Ablation Study

This  ablation  is  not  merely  “remove  a  module  ⇒  score drops.” It isolates a failure mode of guided MARL: when Φ dominates  the  actor, policies  overfit  coordination  surrogates and lose long-horizon task competence. Residual decoupling keeps guidance assistive, explaining the  ~42× Success col- lapse without it. (AC gate ablations appear in Sec. VI-D.)

## C.  Communication Budget Analysis

Under a 90% budget cut (100%→ 10%), AC-DSGF success drops only ~11% relatively (27.1 → 24.2), i.e., graceful degra- dation. Paper wording prefers “over three orders of magnitude” for gate-level soft-cost reductions versus “no-budget” ablation, rather than advertising un-normalized 5000× factors without defining soft cost.

## D.  Silence  Collapse Ablation

Interpretation. Sparsity is not silence collapse. AC-full trades a  few  percentage  points  of  Success  for  over-three-orders reduction in Comm relative to full local broadcast—matching the  locked  claim  of  comparable  performance  at  far  lower messaging.

## E.   Communication Behavior Interpretation

Pearson correlations with Comm: goal distance 0.03, dis- persion  —0.57,  nearest-neighbor  risk  0.84.  Communication intensifies under proximity risk  and  attenuates  when  agents disperse—evidence  of  task-dependent  topology  rather  than random pruning.

## F.  Scalability Analysis

Fig. 6  places  the  Table  I  communication  result  of  AC- DSGF in a scaling context: maintaining task effectiveness does not require paying the dense-graph communication tax as N grows.

## G.  Robustness under Packet Loss

From  p=0  to  p=0.7,  GAT  Success  drops  from   21.2%  to 15.8% (relative  ~25%),  while  AC-DSGF  moves  only  from 26.2% to 25.8%. MAPPO is insensitive by construction (no graph  messaging).  We   interpret  AC-DSGF’s   flat  curve  as graceful degradation: messaging was already sparse and task- conditioned,  so  random  drops  remove  fewer  coordination- critical edges on average.

Fig. 4.    Success vs.  communication budget  (N=16). AC-DSGF maintains task effectiveness under severe budget cuts (graceful degradation).

TABLE VIII

COMMUNICATION MECHANISM VALIDATION (N=16, AC CHECKPOINT) .

OPENING ALL RADIUS EDGES INFLATES COMM MASSIVELY WITH ONLY A

SMALL SUCCESS GAIN .

| Variant | Success (%) | Comm | CEI |
| --- | --- | --- | --- |
| AC-full (learned) | 24.6 | 0.008 | 32.0 |
| AC-no budget (g=A) | 28.9 | 40.6 | 0.007 |
| AC-random gates | 28.5 | 20.3 | 0.014 |

## H.  Communication–Performance Pareto

LIMITATIONS

Absolute  Success  at  N=16  remains  low  for  all  meth- ods; claims are comparative and communication-centric. Bud- get/silence diagnostics may use different episode counts than Table I and must be described as such. Soft gate costs are not identical to packet counts on hardware radios.

# VII.  CONCLUSION

We  presented  AC-DSGF  for  communication-constrained multi-UAV cooperative navigation. By jointly optimizing task return and communication mass E[Σ rt] - λcE[Σ Ct], learn- ing a state-/risk-dependent gate, and enforcing Top-K budgets atop  residual  DSGF  guidance,  AC-DSGF  maintains   task effectiveness while reducing  unnecessary  communication. At N=16, it achieves Success comparable to DSGF with ap- proximately 90× lower communication cost, with supporting budget, silence-collapse, trigger, compute-cost, and packet-loss analyses.  Future  work  includes  hardware  radio  models  and larger-scale field validation.

# REFERENCES

[1]  C. Yu, A. Velu, E. Vinitsky, J. Gao, Y. Wang, A. Bayen, and Y. Wu, “The surprising effectiveness of PPO in cooperative multi-agent games,” in NeurIPS, 2022.

[2]  R. Lowe, Y. Wu, A. Tamar, J. Harb, P. Abbeel, and I. Mordatch, “Multi- agent actor-critic for mixed cooperative-competitive environments,” in NeurIPS, 2017.

[3]  T. Rashid, M. Samvelyan, C. Schroeder, G. Farquhar, J. Foerster, and S. Whiteson, “QMIX: Monotonic value function factorisation for deep multi-agent reinforcement learning,” in ICML, 2018.

[4]  S.-J. Chung, A. A. Paranjape, P. Dames,  S.  Shen,  and V.  Kumar,  “A survey on aerial swarm robotics,” IEEE Trans. Robot., vol. 34, no. 4, pp. 837–855, 2018.

[5]  M.  Bettini,  R.  Kortvelesy,  J.  Blumenkamp,  and  A.  Prorok,  “VMAS: A vectorized multi-agent simulator for collective robot learning,” arXiv preprint arXiv:2207.03530, 2022.

[6]  J. Jiang and Z. Lu, “Learning attentional communication for multi-agent cooperation,” in NeurIPS, 2018.

Fig.  5.    Trigger  analysis:  Comm  correlates  strongly  with  proximity  risk (corr=0.84) and negatively with spatial dispersion (corr=-0.57).

Fig. 6.  Communication versus number of UAVs. Dense / GAT schemas grow quickly; DSGF uses geometric support; AC-DSGF at N=16 sits far lower (Table V), consistent with O(NK) budgeted messaging (Sec. IV-G).

[7]  S. Iqbal and F. Sha, “Actor-attention-critic for multi-agent reinforcement learning,” in ICML, 2019.

[8]  J. Jiang, C. Dun, T. Huang, and Z. Lu, “Graph convolutional reinforce- ment learning,” in ICLR, 2020.

[9]  P.  Velikovi,  G.  Cucurull,  A.  Casanova,  A.  Romero,  P.  Li,  and Y. Bengio, “Graph attention networks,” in ICLR, 2018.

[10]  A. Singh, T. Jain, and S. Sukhbaatar, “Learning when to communicate at scale in multiagent cooperative and competitive tasks,” in ICLR, 2019.

[11]  R. Wang, X. He, R. Yu, W. Qiu, B. An, and Z. Rabinovich,  “Learn- ing  efficient  multi-agent  communication:  An  information  bottleneck approach,” in ICML, 2020.

[12]  J. Foerster, Y. M. Assael, N.  de Freitas,  and  S. Whiteson,  “Learning to  communicate  with  deep  multi-agent  reinforcement  learning,”  in NeurIPS, 2016.

[13]  S. Sukhbaatar, A. Szlam, and R. Fergus, “Learning multiagent commu- nication with backpropagation,” in NeurIPS, 2016.

[14]  A.  Das,  T.  Greeff,  A.  Gupta  et  al.,  “TarMAC:  Targeted  multi-agent communication,” in ICML, 2019.

[15]  D. Kim, S. Moon, D. Hostallero et al., “Learning to schedule commu- nication in multi-agent reinforcement learning,” in ICLR, 2019.

[16]  I.  Mordatch  and  P.  Abbeel,  “Emergence  of  grounded  compositional language in multi-agent populations,” in AAAI, 2018.

7

Fig. 7.   Task  success under random edge drops (N=16, 64-ep diagnostic). AC-DSGF remains nearly flat as loss increases, whereas GAT degrades more noticeably—consistent with graceful degradation under unreliable links.

TABLE IX

SUCCESS (%) VS . PACKET-LOSS RATE p (N=16, SEED 42, 64 EPS) . WE

EMPHASIZE TREND (DEGRADATION SLOPE), NOT ABSOLUTE COMPARISON

WITH TABLE I.

| Method | 0% | 10% | 30% | 50% | 70% |
| --- | --- | --- | --- | --- | --- |
| MAPPO | 31.4 | 30.7 | 31.2 | 33.5 | 29.9 |
| GAT | 21.2 | 17.6 | 16.1 | 14.6 | 15.8 |
| DSGF | 21.8 | 24.2 | 20.7 | 21.5 | 18.5 |
| AC-DSGF | 26.2 | 26.1 | 26.4 | 28.3 | 25.8 |

Fig.  8.   Communication–performance  Pareto  (N=16,  Table V).  AC-DSGF occupies the high-efficiency region: comparable Success, minimal Comm.